# Critical Design Review - Electrical

INNEX-1 Rover - Innova UoL

University of Leicester, UK Lunabotics Team

| Team Name | Innova UoL |
| --- | --- |
| Project Name | INNEX-1 Rover |
| Sub-Team | Electrical |
| Document Title | CDR - Electrical |
| University | University of Leicester |
| Department | School of Engineering |
| Date of Submission | 12/03/2026 |
| Author(s) | Enio Mecaj |
| Authorised by | Tire Joseph |
| Version | 2.2 |

# Summary The Electrical Critical Design Review (CDR) details an electrical architecture centred around a dual-domain power system. By physically isolating the motive domain, powered by a 22.2V 6S 8000mAh LiPo battery, from the compute domain, powered by a 14.8V 4S 5000mAh LiPo battery, the design prevents high-torque voltage drop from browning out the NVIDIA Jetson and sensors. Safety and reliability are heavily prioritised; the system features a 250A E-Stop (push-break, 80V DC rated) that severs motive power while keeping the compute rail and telemetry feeds alive and uses a passive 95dB low-voltage buzzer rather than an active BMS to prevent automated shutdown during heavy mechanical manoeuvres. Furthermore, by enforcing an operational rule that prevents driving and excavating at the same time, the absolute peak current is strictly capped at 40A, giving the main battery a safety factor of 3. This electrical hardware is packaged inside a 5mm polycarbonate enclosure using a single-direction airflow strategy that pulls cool air over the compute modules before exhausting heat from the motor drivers. Finally, the entire system is mapped with load-calculated wire gauges and is backed by a £1,218 electrical budget.

Summary	0

1	Final schematic	4

1.1	Compute Domain Schematic	4

1.2	Power Domain Schematic	4

1.3	Low-Voltage Warning Batteries	5

1.4	Connector Types	5

2	Harness design	18

2.1	Bus Voltage	18

2.2	Wiring Topology:	20

2.2.1	Main Power Domain:	20

2.2.2	Compute Power Domain	21

2.3	Peak Current Calculations	21

2.4	Component Connection Map	21

2.4.1	Main Battery → ANL Fuse → Arming Switch → Power Meter → E-STOP → PDB	21

2.4.2	Power wiring – PDB to Sabertooth	22

2.4.3	Signal wiring – Sabertooth to Jetson	23

2.4.4	Motor Wiring – Sabertooth to GR-WM4-V3 motors	24

2.4.5	Encoder Wiring – GR-QM4-V3 to Jetson	24

2.4.6	Power Wiring – PDB to Cytron	25

2.4.7	Signal Wiring - Cytron to Jetson	25

2.4.8	Actuator Wiring – Cytron to Actuator	26

2.4.9	Power Wiring – PDB to BLD-510B BLDC Controller	27

2.4.10	Signal Wiring – BLD-510B BLDC to Jetson	27

2.4.11	Excavation Motor Wiring – BLDC to Excavation Motor	28

2.5	Complete Wiring Gauge	29

3	Power budget	30

3.1	Drivetrain and excavation subsystem:	31

3.1.1	Scenario A: Driving & Navigating	31

3.1.2	Scenario B: Excavation (Rover is stationary)	31

3.2	Computational subsystem:	31

3.3	Battery C-Rating Numerical Justification	31

3.4	Secondary Compute Battery (14.8V) C-Rating Justification	32

3.5	Battery Runtime Calculation	32

4	Physical integration	33

4.1	Overview	33

4.2	Electrical Layout Architecture	33

4.3	Mechanical Packaging	34

4.4	Thermal Management Strategy	34

4.5	Electrical Safety Considerations	34

4.6	CAD Integration & System Validation	35

4.7	Evidence	35

5	E-STOP & safety chain	36

5.1	Back EMF	36

5.2	Bench Test	37

5.3	E-stop requirements	37

5.4	E-Stop Schematic	38

6	Procurement locked	38

6.1	Main parts BOM	38

6.2	Cost	39

6.3	Alternative List	40

Table of Figures

Figure 1 Compute Domain Schematic	4

Figure 2 Power Domain Schematic	5

Figure 3 Power Domain Diagram	19

Figure 4 Compute Domain Diagram	20

Figure 5 CAD Files for Electrical Box	33

Figure 6 Exhaust and Cooling Diagram	34

Figure 7 E- Stop and Wiring Schematic	36

# Final schematic

## Compute Domain Schematic

Figure 1 Compute Domain Schematic

## Power Domain Schematic

Figure 2 Power Domain Schematic

## Low-Voltage Warning Batteries

The architecture uses a passive low-voltage warning system instead of an active onboard Battery Management System (BMS) to manage battery safety during high-load movement/excavation operations without sacrificing motive power. When high-amperage robotic drivetrains come into contact with obstacles, they often experience brief voltage drops. Unwanted system brownouts and lost autonomy cycles would result from an in-line BMS interpreting these brief drops as critical faults and physically cutting power.

The motive and compute batteries' balance leads are directly connected to 1S-8S LiPo buzzer alarms in order to stop this. These devices independently track the voltages of each cell in real time. A 95dB audible alarm is set off if a single cell falls below the 3.4V safe threshold. By warning operators and scrutineers about a draining cell while keeping the power circuit closed so the operator can complete the current manoeuvre before starting a safe manual shutdown, this passive audio-telemetry meets the competition LiPo safety requirements.

1-8 S Buzzer Low Voltage Alarm Indicator Lipo Batteries : https://www.amazon.co.uk/zalati-Battery-Testers-Voltage-Reminders/dp/B0CJV4RW2T?source=ps-sl-shoppingads-lpcontext&ref_=fplfs&psc=1&smid=A1DHSBK7H5KQMH

## Connector Types

Power Domain Connector Types & Connections

The power domain is supplies by the main battery which is a Turngy 6S LiPo battery (XT90 connector). All the drivetrain motors, actuator motors and the excavation motor and their controllers draw from this rail via the power distribution board (PDB). The safety chain runs – Main battery (+) → 40A inline fuse → Arming switch → E-STOP → PDB.

MAIN BATTERY CHAIN:

| Circuit ID | From | To | AWG | Connector Type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| PW-01 | Main Battery (+) | Inline fuse | 10 AWG | XT90 Anti-spark (battery end) | 40A ANL | Red wire. XT90 anti-spark connector at the battery terminal. |
| PW-02 | ANL fuse out | Arming switch L1 | 10 AWG | Ring terminal on switch lugs |  | Red wire. Ring terminal crimped to arming switch lugs. Inline ANL fuse output. |
| PW-03 | Power meter +OUT | E-STOP terminal A | 10 AWG | High-Current ring terminal on E-STOP input. |  | After power meter. |
| PW-04 | E-STOP terminal B | PDB +IN | PDB +IN | Direct wire into PDB positive rail |  | E-STOP output into PDB positive bus. Final link before PDB |
| PW-GND | Main Battery (-) | PDB -IN | PDB -IN | Direct wire into PDB negative rail |  | Black wire. Ground BYPASSES E-STOP – direct battery negative to PDB ground bus. |

POWER WIRING: PDB to Sabertooth motor controllers

| Circuit ID | From | To | AWG | Connector Type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| MC-01 + | PDB + Rail | Sabertooth #1 B+ | 10 AWG | Ring Terminal on Sabertooth screw terminal | None | Fuse holder within 50mm of PDB output. Red wire into Sabertooth B+ screw terminal. |
| MC-01 - | PDB – Rail | Sabertooth #1 B- | 10 AWG | Screw Terminal (direct) |  | Black wire into Sabertooth B- screw terminal. No fuse on return path. |
| MC-02 + | PDB + Rail | Sabertooth #2 B+ | 10 AWG | Ring terminal on sabretooth screw terminal | None | Separate fuse per controller. Fuse holder within 50mm of PDB output. |
| MC-02 - | PDB – Rail | Sabertooth #2 B- | 10 AWG | Direct wire into PDB negative rail |  | Black wire. Ground return for sabretooth #2 |

SIGNAL WIRING: Sabertooth motor controllers to Jetson Orin Nano

| Signal | From (Sabertooth pin) | To (Jetson Orin Nano) | AWG | Connector Type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| S1 (Serial) | S1 Terminal | Teensy 4.1 UART TX | 24 AWG | Dupont/JST (logic level signal wire) |  | Simplified serial (one wire per controller). DIP switches set to Serial mode. 9600 baud default. |
| 0V (GND) | 0V Terminal | Common GND Bus | 24 AWG | Dupont/JST (signal ground) |  | MUST share ground with Jetson for correct signal reference. Essential for signal integrity. |

MOTOR WIRING: Sabertooth to GR-WM4-V3

| Circuit ID | From | To | AWG | Connector Type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| MT-01 | Sabertooth #1 M1A/M1B | Drive motor 1 +/- | 14AWG | Screw Terminal (sabretooth) → GR-WM4 6P Latch Bullet Connector. |  | 10A/channel (software limited via Sabertooth). Screw terminal exit from Sabertooth, transitioning to 6P latch bullet on GR-WM4 harness. |
| MT-02 | Sabertooth #1 M2A/M2B | Drive motor 2 +/- | 14AWG | Screw Terminal (sabretooth) → GR-WM4 6P Latch Bullet Connector. |  | 10A/channel (software limited via Sabertooth). Screw terminal exit from Sabertooth, transitioning to 6P latch bullet on GR-WM4 harness. |
| MT-01 | Sabertooth #2 M1A/M1B | Drive motor 3 +/- | 14AWG | Screw Terminal (sabretooth) → GR-WM4 6P Latch Bullet Connector. |  | 10A/channel (software limited via Sabertooth). Screw terminal exit from Sabertooth, transitioning to 6P latch bullet on GR-WM4 harness. |
| MT-01 | Sabertooth #2 M2A/M2B | Drive motor 4 +/- | 14AWG | Screw Terminal (sabretooth) → GR-WM4 6P Latch Bullet Connector. |  | 10A/channel (software limited via Sabertooth). Screw terminal exit from Sabertooth, transitioning to 6P latch bullet on GR-WM4 harness. |

ENCODER WIRING: GR-QM4-V3 Encoders to Jetson

| Wire | Colour | From | AWG | Connector Type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| VCC | Red | Teensy 4.1 3.3V | 26AWG | WAGO 221-413 (shared 5V supply) |  | 5V encoder power. <10mA. Can share supply with WAGO 221-413 splitter. |
| GND | Black | Common GND Bus | 26AWG | WAGO/Dupont (signal ground) |  | Teensy GND. Keep encoder GND on separate WAGO output from motor controller GNDs to avoid noise coupling. |
| CH A | Blue | Teensy 4.1 GPIO | 26AWG | Dupont/JST |  | Twist CH A & CH B together. Route away from 14 AWG motor wires to prevent interference. |
| CH B | Yellow | Teensy 4.1 GPIO | 26AWG | Dupont/JST |  | Quadrature pair with CH A. Route twisted with CH A. |
| VCC | Red | Teensy 4.1 3.3V | 26AWG | WAGO 221-413 (4th encoder) |  | 5V encoder power. <10mA. Can share supply with WAGO 221-413 splitter. |

POWER WIRING: PDB to Cytron Actuator motor controller

| Circuit ID | From | To | AWG | Connector type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ACT-01 + | PDB + Rail | Cytron #1 B+ | 10 AWG | Ring Terminal on Cytron Screw Terminal | 15A Inline | Fuse holder within 50mm of PDB output. Red wire into Cytron B+ screw terminal. |
| ACT-01 - | PDB − Rail | Cytron #1 B− | 10 AWG | Ring Terminal on Cytron Screw Terminal |  | Black wire to Cytron B− terminal. No fuse on return path. |
| ACT-02 + | PDB + Rail | Cytron #2 B+ | 10 AWG | Ring Terminal on Cytron Screw Terminal | 15A Inline | Fuse holder within 50mm of PDB output. Red wire into Cytron B+ screw terminal. |
| ACT-02 - | PDB − Rail | Cytron #2 B− | 10 AWG | Ring Terminal on Cytron Screw Terminal |  | Black wire to Cytron B− terminal. No fuse on return path. |

SIGNAL WIRING: Cytron motor controller to Jetson

| Signal | From (cytron pin) | To (jetson) | AWG | Connector Type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| PWN 1 | PWM 1 Pin | Teensy 4.1 PWM | 24 AWG | Dupont/JST (logic signal) |  | Teensy 4.1 native PWM (Pin 2 or 3 for Cytron #1, Pin 4 or 5 for Cytron #2). No I2C expander needed. |
| DIR 1 | DIR 1 Pin | Teensy 4.1 GPIO | 24 AWG | Dupont/JST (logic signal) |  | Standard 3.3V logic. Teensy GPIO: Pin 9/10 for Cytron #1, Pin 11/12 for Cytron #2. |
| PWM 2 | PMW 2 pin | Teensy 4.1 PWM | 24 AWG | Dupont/JST (logic signal) |  | Teensy 4.1 native PWM (Pin 2 or 3 for Cytron #1, Pin 4 or 5 for Cytron #2). No I2C expander needed. |
| DIR 2 | DIR 2 Pin | Teensy 4.1 GPIO | 24 AWG | Dupont/JST (logic signal) |  | Standard 3.3V logic. Teensy GPIO: Pin 9/10 for Cytron #1, Pin 11/12 for Cytron #2. |
| GND | GND Pin | Common GND Bus | 24 AWG | Dupont/JST (logic signal) |  | MUST share ground with Teensy 4.1. |

ACTUATOR WIRING: Cytron to Linear Actuators

| Circuit ID | From | To | AWG | Connector type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ACT-M-01 | Cytron #1 M1A/M1B | TRU 50mm Actuator 1 +/- (Brown+/Blue-) | 14 AWG | Screw Terminal (Cytron) → Actuator Power Leads |  | TRU Components 50mm. Cytron #1 M1, 22.2V direct. Brown(+)/Blue(-). 1.6A peak. Polarity reverses for extend/retract. |
| ACT-M-02 | Cytron #1 M2A/M2B | TRU 50mm Actuator 2 +/- (Brown+/Blue-) | 14 AWG | Screw Terminal (Cytron) → Actuator Power Leads |  | TRU Components 50mm. Cytron #1 M2, 22.2V direct. Brown(+)/Blue(-). 1.6A peak. Polarity reverses for extend/retract. |
| ACT-M-03 | Cytron #2 M1A/M1B | DCHOUSE 250mm Actuator 3 +/- (Red+/Black-) | 14 AWG | Screw Terminal (Cytron) → Actuator Power Leads |  | DCHOUSE 250mm. Cytron #2 M1, 12V via DollaTek buck. Red(+)/Black(-). 3A peak. Polarity reverses for extend/retract. |
| ACT-M-04 | Cytron #2 M2A/M2B | DCHOUSE 250mm Actuator 4 +/- (Red+/Black-) | 14 AWG | Screw Terminal (Cytron) → Actuator Power Leads |  | DCHOUSE 250mm. Cytron #2 M2, 12V via DollaTek buck. Red(+)/Black(-). 3A peak. Polarity reverses for extend/retract. |

POWER WIRING: PDB to BLD-510B BLDC Excavation Controller

| Circuit ID | From | To | AWG | Connector Type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| EXC-PWR + | PDB + rail | BLDC VCC (B+) | 16 AWG | Screw Terminal on BLDC Controller | 10A blade | Main 22.2V power for excavation system. Blade fuse in holder. |
| EXC-PWR - | PDB - rail | BLDC GND (B-) | 16 AWG | Screw Terminal on BLDC Controller |  | Ground return for excavation controller. No fuse on return path. |

SIGNAL WIRING: BLD-510B BLDC Controller to Jetson

| Signal | From (BLDC Pin) | To (Jetson Pin) | AWG | Connector type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| EXC-SIG-VR | VR (speed) | Teensy Pin 6 | 24 AWG | Dupont/JST (logic signal) |  | Teensy Pin 6 PWM → 10kΩ + 10µF RC filter → BLD-510B SV pin. Converts PWM to 0–5V analog for speed control. |
| EXC-SIG-DIR | F/R (Direction) | Teensy Pin 13 | 24 AWG |  |  | Teensy Pin 13 GPIO. Toggles forward/reverse. BLD-510B EN (enable) on Teensy Pin 14. |
| EXC-SIG-GND | GND | Common GND Bus | 24 AWG |  |  | Logic ground reference. MUST connect to jetson ground. |

MOTOR WIRING: BLD-510B BLDC Controller to 57BLR50 Excavation Motor

| Circuit ID | From (BLDC out) | To (Motor wire) | AWG | Connector type | Fuse | notes |
| --- | --- | --- | --- | --- | --- | --- |
| EXC-MOT-U | BLDC U | Phase U (Orange) | 18 AWG | 4mm bullet connector |  | Match to motor orange wire |
| EXC-MOT-V | BLDC V | Phase V (Green) | 18 AWG | 4mm bullet connector |  | Match to motor green wire |
| EXC-MOT-W | BLDC W | Phase W (Brown) | 18 AWG | 4mm bullet connector |  | Match to motor brown wire |
| EXC-HALL-5V | BLDC 5V OUT | Hall Sensor 5V (Red) | 26 AWG | Direct solder / JST |  | Power internal hall sensor |
| EXC-HALL-GND | BLDC GND | Hall sensor GND (Black) | 26 AWG | Direct solder / JST |  | Hall sensor ground reference |
| EXC-HALL-A | BLDC Ha | Hall A (Yellow) | 26 AWG | Direct solder / JST |  | Positive feedback wire A |
| EXC-HALL-B | BLDC Hb | Hall B (White) | 26 AWG | Direct solder / JST |  | Positive feedback wire B |
| EXC-HALL-C | BLDC Hc | Hall C (Blue) | 26 AWG | Direct solder / JST |  | Positive feedback wire C |

Power

The compute domain is supplied by the secondary Turnigy 4S 5000mAh LiPo battery (XT90 connector). This domain is fully isolated from the main power domain (22.2V). this is done to protect sensitive electronics. Power is stepped down by 2 buck converters: BD-01 (DollaTek Reg.NO:013726393, 14.8V → 12V, ≤10A) for Jetson and LiDAR, and BD-02 (Yosoo Health Gear g0qigxo64d, 14.8V → 5V, ≤10A) for cameras, router and IMU (distributed via WAGO 2-in-6-out block). The Teensy 4.1 is powered directly from the Jetson via USB (5V, ~0.5A max). Encoders are powered from the Teensy 4.1 3.3V rail.

SECONDARY BATTERY CHAIN

| Circuit ID | From | To | AWG | Connector Type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| SEC-01 | Secondary battery (14.8V +) | Buck converter BD-01 (12V in) & BD-02 (5V in) | 18 AWG |  | 10A blade | 3A peak. 14.8V source to both buck converters. Fully isolated from 22.2V main power domain. |
| SEC-GND | Secondary battery (-) | Common Compute GND Bus | 18 AWG |  |  | Compute domain ground. Isolated from the main power domain ground. |

COMPUTE POWER: Buck converter BD-01 to Jetson orin nano

| Circuit ID | From | To | AWG | Connector type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| BD-01 IN | Secondary battery (+) | Buck BD-01 input | 18 AWG | XT60 (Shared with SEC-01) | 10A blade | 14.8V input to DollaTek (Reg.NO:013726393) step-down module (12V output). Rated ≤10A — Jetson (~4A) + OS1 LiDAR (~3A) = ~7A combined. 12V output distributed via WAGO 2-in-4-out block. |
| BD-01 OUT | Buck output (12V) | Jetson Orin Nano Power Jack | 20 AWG | Barrel Jack – OD 5.5mm / ID 2.5mm (DC Jack) |  | 1.3A peak. XL4015 output adjusted to 12V. Jetson accepts 9-19V DC input via barrel jack. Also feeds Ouster OS1 LiDAR (12V, ~2-3A). |

COMPUTE POWER: Buck converter BD-02 (14.8V to 5V) to router, cameras and IMU. (Encoders: Teensy 4.1 3.3V rail)

| Circuit ID | From | To | AWG | Connector type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| BD-01 in | Secondary battery (+) | Buck BD-02 input | 18 AWG | Solder Pads (XL4015 module) / XT30 shared | 5A Blade | 14.8V in; BD-02 (Yosoo Health Gear g0qigxo64d) adjusted to 5V output. ≤10A capability. Output distributed via WAGO 2-in-6-out block (3 active outputs: router, camera ×2). |
| BD02-OUT → Router | Buck BD-02 Output (5V) | GL-iNet Slate Plus Router | 20 AWG | USB-C |  | 3A peak (15W max). Must run on 2.4 GHz band only. USB-C power delivery. |
| LP-LDR | 5V Bus (BD-02 Out) | Ouster OS1 LiDAR | 22 AWG | GH1.25 4-pin (via adapter module) |  | ⚠️ Powered from BD-01 12V rail (not BD-02 5V). ~2-3A. GH1.25 4-pin connector. OS1 connects via GL-A1300 router LAN → Jetson GbE. |
| LP-CAM | 5V Bus (BD-02 Out) | OAK-D Pro Cameras (x2) | 22 AWG | USB-C (each camera) |  | 0.9A per camera peak (2A max each). USB-C power and data combined on single cable per camera. |
| LP-IMU | 5V Bus (BD-02 Out) | IMU Module | 24 AWG | WAGO 221-413 / UART Pin Header |  | 0.1A peak. Logic-level 5V supply. UART data line runs separately to Jetson GPIO. |
| ENC-01-04 | Teensy 4.1 3.3V | Motor Encoders (x4 drive + 1 excav.) | 26 AWG | Dupont 4-wire (VCC from Teensy 3.3V pin) |  | 0.01A per encoder (<10mA each). 4-wire harness: VCC (Red), GND (Black), CH A (Blue), CH B (Yellow). |

SIGNAL WIRING: Jetson Orin Nano to command controllers

| Circuit ID | From (Jetson) | To (Controller / Sensor) | AWG | Connector Type | Fuse | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| SIG-UART-SAB1 | Teensy 4.1 UART TX | Sabertooth #1 S1 Terminal | <0.5A | Dupont / JST (signal) |  | Teensy UART TX → Sabertooth S1. Packetised serial 9600 baud. Jetson sends commands to Teensy via USB serial; Teensy relays to Sabertooth. |
| SIG-UART-SAB2 | Teensy 4.1 UART TX | Sabertooth #2 S1 Terminal | <0.5A | Dupont / JST (signal) |  | Teensy UART TX → Sabertooth S1. Packetised serial 9600 baud. Jetson sends commands to Teensy via USB serial; Teensy relays to Sabertooth. |
| SIG-PWM-CYT1 | Teensy 4.1 PWM | Cytron #1 PWM1/PWM2 | <0.5A | Dupont / JST (signal) |  | Teensy 4.1 native PWM. Controls actuators 1 & 2. Pins 2 (PWM1) and 3 (PWM2). |
| SIG-DIR-CYT1 | Jetson GPIO (3.3V) | Cytron #1 DIR1/DIR2 | <0.5A | Dupont / JST (signal) |  | Digital direction control. Logic-level 3.3V. |
| SIG-PWM-CYT2 | Teensy 4.1 PWM | Cytron #2 PWM1/PWM2 | <0.5A | Dupont / JST (signal) |  | Teensy 4.1 native PWM. Controls actuators 3 & 4. Pins 4 (PWM1) and 5 (PWM2). |
| SIG-DIR-CYT2 | Jetson GPIO (3.3V) | Cytron #2 DIR1/DIR2 | <0.5A | Dupont / JST (signal) |  | Digital direction control. Logic-level 3.3V. |
| SIG-PWM-BLDC | Teensy Pin 6 (PWM) | Teensy Pin 6 (PWM via RC filter) | <0.5A | Dupont / JST (signal) |  | Teensy Pin 6 PWM → RC filter → BLD-510B SV pin. |
| SIG-DIR-BLDC | Teensy Pin 13 (GPIO) | Teensy Pin 13 (GPIO) | <0.5A | Dupont / JST (signal) |  | Toggles forward/reverse on excavation motor. |
| SIG-ENC-CHx | Teensy GPIO (Pins 15-22) | Encoder CH A & CH B (x4) | <0.5A | Dupont / JST (signal) |  | Teensy Pins 15-22 (FL:15/16, RL:17/18, FR:19/20, RR:21/22). Twisted-pair 26AWG routed away from motor wires. |
| SIG-CAM-USB | Jetson USB 3.0 Port | OAK-D Pro Cameras (x2) | <0.5A | Dupont / JST (signal) |  | USB 3.0 SuperSpeed. Data + power on USB-C cable per camera. |
| SIG-LIDAR | GL-A1300 Router LAN port | Ouster OS1 LiDAR | <0.5A | Dupont / JST (signal) |  | Gigabit Ethernet to Jetson. Power 12V from BD-01 rail (~2-3A). 60s boot time before data available. |
| SIG-ROUTER | Jetson Ethernet | GL-iNet Slate Plus Router | <0.5A | Dupont / JST (signal) |  | Gigabit Ethernet. Telemetry stream must remain under 4,000 Kbps. |

# Harness design

## Bus Voltage

The main battery is the Turnigy Professional 8000mAh 6S 15C LiPo w/XT90. This battery gives us a nominal bus voltage of 22.2V but when fully charged the voltage is 25.2V. All the motors, motor controllers and the PDB draw power from this battery directly.

The secondary battery is Turnigy 4S 5000mAh 14.8V 40C Lipo Battery. This battery will supply power to the low voltage electronic components such as the router, jetson, LiDAR, and cameras via our voltage regulators.

| Component | Qty | Power Rail | Connector | Notes |
| --- | --- | --- | --- | --- |
| Main Battery | 1 | Source 22.2V | XT90 | 8Ah * 15C = 120A discharge |
| Secondary Battery | 1 | Source 14.8 V | XT90 | 5Ah * 40C = 200 A discharge |
| Power Distribution Board | 1 | 24V in | Screw Terminal | Central hub for all power |
| Sabertooth 2x32 (drivetrain) | 2 | 22.2V (6-30V) | Screw terminals | 32A constant, 64A peak per channel |
| Drivetrain motor | 4 | 22.2V DC | Bullet/6P connector | 10A/channel (software limited) |
| Excavation motor | 1 | 22.2V DC | Bullet terminals |  |
| Voltage regulator (Jetson) | 1 | 14.8V in 12V out | Solder pads | Max 10A output |
| Voltage regulator (Jetson) | 1 | 14.8V in 5V out | Screw Terminals | Max 10A output |
| Jetson Orin Nano Developer Kit | 1 | 9 - 19V | DC Jack OD 5.5mm ID 2.5mm | 3A |
| Router | 1 | 5V 3A (max 15W) | USB-C | Must run on 2.4 GHz only |
| LiDAR | 1 | 12V DC | GH1.25, 4-pin (uses adapter module) | ~500mA |
| OAK-D Camera | 2 | 5V (USB-C) | USB-C | 2A |
| Motor encoders x4 | 5 | 5V DC | 4-wire | ~10mA each |

## Wiring Topology:

### Main Power Domain:

Figure 3 Power Domain Diagram

### Compute Power Domain

Figure 4 Compute Domain Diagram

## Peak Current Calculations

Using the datasheets: find the peak current for the component for example the GR-WM4-V3 the datasheet states:

- Stall current at 22.2V: ~22.6A

- Current at peak gearbox torque: 13A

- Apply the data sheets 10% safety factor: 13A x 1.10 = 14.3A

The peak current can also be calculated from the power – use: I = P ÷ V . For example the router at 5V, max 15W: I = 15 ÷ 5 which equals = 3.0A.

Circuit/Wiring ID’s: PT = Power Trunk, MC = Motor controllers, MT = Motor terminal, BC = Buck Converters, LP = Low Power, ENC = Encoders.

## Component Connection Map

### Main Battery → ANL Fuse → Arming Switch → Power Meter → E-STOP → PDB

| Circuit ID | From | To | AWG | Fuse | Connector/Notes |
| --- | --- | --- | --- | --- | --- |
| PW-01 | Main Battery + | ANL Fuse IN | 10 AWG | 40A ANL | XT90 anti-spark at battery. Red wire. |
| PW-02 | ANL Fuse OUT | Arming Switch L1 | 10 AWG |  | Ring terminals on switch lugs. Red wire. |
| PW-03 | Arming Switch L2 | Power Meter +IN | 10 AWG |  | High-current ring terminal on E-STOP stud. |
| PW-04 | Power Meter +OUT | E-STOP Terminal A | 10 AWG |  | High-current ring terminal onto E-STOP input lug. |
| PW-05 | E-STOP Terminal B | PDB +IN | 10 AWG |  | Wire into PDB positive rail. |
| PW-GND | Main Battery - | PDB -IN | 10 AWG |  | Black wire. Ground does NOT pass through E-STOP. |

### Power wiring – PDB to Sabertooth

| Circuit ID | From | To | AWG | Fuse | Notes |
| --- | --- | --- | --- | --- | --- |
| MC-01 + | PDB + rail | Sabertooth #1 B+ | 10 AWG | None | Fuse within 50mm of PDB output. Ring terminal |
| MC-01 - | PDB – rail | Sabertooth #1 B- | 10 AWG |  | Black wire to Sabertooth B- terminal |
| MC-02 + | PDB + rail | Sabertooth #2 B+ | 10 AWG | None | PDB 15A fuse → DollaTek 300W buck (22.2V→12V) → Cytron #2 VPWR. 250mm actuators rated 12V. |
| MC-02 - | PDB – rail | Sabertooth #2 B- | 10 AWG |  | Black wire |

### Signal wiring – Sabertooth to Jetson

Using Simplified Serial (one wire per controller). Set DIP switches on Sabertooth to Serial mode.

| Signal | Sabertooth Pin | Connects To | AWG | Notes |
| --- | --- | --- | --- | --- |
| S1 (Serial) | S1 terminal | Teensy 4.1 UART TX | 24 AWG | Single-wire simplified serial. 9600 baud default |
| 0V (Ground) | 0V terminal | Common GND Bus | 24 AWG | MUST share ground with Jetson. Essential for signal reference |
| 5V OUT | 5V terminal | Do NOT connect |  | Sabertooth can supply 5V but use your BC-02 instead for cleaner power |

### Motor Wiring – Sabertooth to GR-WM4-V3 motors

| Circuit ID | From | To | AWG | Connector/Notes |
| --- | --- | --- | --- | --- |
| MT-01 | Sabertooth #1 M1A/M1B | Drive Motor 1 +/- | 14 AWG | Screw terminal on Sabertooth → bullet connector of GR-WM4 6P latch connector |
| MT-02 | Sabertooth #1 M2A/M2B | Drive Motor 2 +/- | 14 AWG | Same as MT-01 |
| MT-03 | Sabertooth #2 | Drive Motor 3 +/- | 14 AWG | Same as MT-01 |
| MT-04 | Sabertooth #2 | Drive Motor 4 +/- | 14 AWG | Same as MT-01 |

### Encoder Wiring – GR-QM4-V3 to Jetson

| Wire | Colour | Connects to | AWG | Notes |
| --- | --- | --- | --- | --- |
| VCC | Red | Teensy 4.1 3.3V | 26AWG | 5V encoder power. <10mA. Can use WAGO 221-413 to share supply |
| GND | Black | Common GND bus | 26 AWG | Share ground with Teensy |
| CH A | Blue | Jetson GPIO input | 26 AWG | Twist CH A and CH B together. Route away from 14 AWG motor wires |
| CH B | Yellow | Jetson GPIO | 26 AWG | Quadrature pair with CH A |

### Power Wiring – PDB to Cytron

| Circuit ID | From | To | AWG | Fuse | Notes |
| --- | --- | --- | --- | --- | --- |
| ACT-01 + | PDB + rail | Cytron #1 B+ | 10 AWG | 15A Inline | Fuse within 50mm of PDB output. Ring terminal |
| ACT-01 - | PDB – rail | Cytron #1 B- | 10 AWG |  | Black wire to Cytron B- terminal |
| ACT-02 + | PDB + rail | Cytron #2 B+ | 10 AWG | 15A Inline | PDB 15A fuse → DollaTek 300W buck (22.2V→12V) → Cytron #2 VPWR. 250mm actuators rated 12V. |
| ACT-02 - | PDB – rail | Cytron #2 B- | 10 AWG |  | Black wire |

### Signal Wiring - Cytron to Jetson

| Signal | Cytron Pin | Connects To | AWG | Notes |
| --- | --- | --- | --- | --- |
| PWM 1 | PWM1 Pin | Teensy 4.1 PWM | 24 AWG | Teensy 4.1 native PWM. PCA9685 I2C expander removed — Teensy has 31 native PWM pins. |
| DIR 1 | DIR1 Pin | Teensy 4.1 GPIO | 24 AWG | Standard 3.3V logic. Teensy GPIO: Pin 9/10 for Cytron #1, Pin 11/12 for Cytron #2. |
| PWM 2 | PWM2 Pin | Teensy 4.1 PWM | 24 AWG | Teensy 4.1 native PWM. PCA9685 I2C expander removed — Teensy has 31 native PWM pins. |
| DIR 2 | DIR2 Pin | Teensy 4.1 GPIO | 24 AWG | Standard 3.3V logic. Teensy GPIO: Pin 9/10 for Cytron #1, Pin 11/12 for Cytron #2. |
| GND | GND Pin | Common GND Bus | 24 AWG | MUST share ground with Teensy 4.1. |

### Actuator Wiring – Cytron to Actuator

| Circuit ID | From | To | AWG | Connector/Notes |
| --- | --- | --- | --- | --- |
| ACT-M-01 | Cytron #1 M1A/M1B | TRU 50mm Actuator 1 +/- (Brown+/Blue-) | 14 AWG | TRU Components 50mm. Cytron #1 M1, 22.2V direct. Brown(+)/Blue(-). 1.6A peak. Polarity reverses for extend/retract. |
| ACT-M-02 | Cytron #1 M2A/M2B | TRU 50mm Actuator 2 +/- (Brown+/Blue-) | 14 AWG | Same as MT-01 |
| ACT-M-03 | Cytron #2 | DCHOUSE 250mm Actuator 3 +/- (Red+/Black-) | 14 AWG | Same as MT-01 |
| ACT-M-04 | Cytron #2 | DCHOUSE 250mm Actuator 4 +/- (Red+/Black-) | 14 AWG | Same as MT-01 |

### Power Wiring – PDB to BLD-510B BLDC Controller

| Circuit ID | From | To | AWG | Fuse | Notes |
| --- | --- | --- | --- | --- | --- |
| EXC-PWR + | PDB + rail | BLDC VCC (B+) | 16 AWG | 10A Blade | Main 22.2V power for excavation. Screw terminal. |
| EXC-PWR - | PDB - rail | BLDC GND (B-) | 16 AWG |  | Ground return. Screw terminal. |

### Signal Wiring – BLD-510B BLDC to Jetson

| Signal | Cytron Pin | Connects To | AWG | Notes |
| --- | --- | --- | --- | --- |
| EXC-SIG-VR | VR (Speed) | Teensy Pin 6 | 24 AWG | Teensy Pin 6 PWM → 10kΩ + 10µF RC filter → BLD-510B SV pin. Converts PWM to 0–5V analog for speed control. |
| EXC-SIG-DIR | Z/F (Direction) | Teensy Pin 13 | 24 AWG | Teensy Pin 13 GPIO. F/R direction on BLD-510B. |
| EXC-SIG-GND | GND | Common GND Bus | 24 AWG | Logic ground reference. MUST connect to Jetson ground. |

### Excavation Motor Wiring – BLDC to Excavation Motor

| Circuit ID | From | To | AWG | Connector/Notes |
| --- | --- | --- | --- | --- |
| EXC-MOT-U | BLDC U | Phase U (Orange) | 18 AWG | 4mm Bullet. Match to motor's ORG wire. |
| EXC-MOT-V | BLDC V | Phase V (Green) | 18 AWG | 4mm Bullet. Match to motor's GRN wire. |
| EXC-MOT-W | BLDC W | Phase W (Brown) | 18 AWG | 4mm Bullet. Match to motor's BRW wire. |
| EXC-HALL-5V | BLDC 5V | Hall 5V (Red) | 26 AWG | Powers internal sensors. |
| EXC-HALL-GND | BLDC GND | Hall GND (Black) | 26 AWG | Sensor ground. |
| EXC-HALL-A | BLDC Ha | Hall A (Yellow) | 26 AWG | Position feedback wire A. |
| EXC-HALL-B | BLDC Hb | Hall B (White) | 26 AWG | Position feedback wire B. |
| EXC-HALL-C | BLDC Hc | Hall C (Blue) | 26 AWG | Position feedback wire C. |

## Complete Wiring Gauge

| ID | Circuit | Peak I (A) | +10% | =0.8 Req. | AWG | Capacity | Fuse | Connector |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PT-01-05 | Batt trunk (all) | 40 | 44 | 55A | 6 | 75A | 40A ANL | XT90 |
| MC-01 +/- | PDB → Sabertooth #1 | 26 | 28.6 | 35.8A | 10 | 40A | 40A blade | Screw |
| MC-02 +/- | PDB → Sabertooth #2 | 26 | 28.6 | 35.8A | 10 | 40A | 40A blade | Screw |
| EX- 01+/- | PDB → RoboClaw |  |  |  | 14 | 20A | 20A blade | Screw |
| MT-01-04 | Sabertooth → Drv Motor | 13 | 14.3 | 17.9 | 14 | 20A |  | Bullet/6P |
| EX-02 | RoboClaw → Exc Motor |  |  |  | 14 | 20A |  | Bullet/Lug |
| BC01-IN | PDB → Buck 12V | 16 | 17.6 | 22A | 18 | 10A | 20A blade | Solder pads |
| BC02- IN | PDB → Buck 5V | 2.5 | 2.75 | 3.4A | 20 | 7.5A | 5A blade | Solder pads |
| ACT- 01-04 | Buck 12V → Actuators | 4 ea | 4.4 | 5.5A | 20 | 7.5A |  | WAGO 221-415 |
| LP-LDR | BD-01 12V → LiDAR | 0.5 | 0.55 | 0.69A | 22 | 5A |  | WAGO/GH1.25 |
| LP-CAM | 5V bus →OAK-D | 0.9 | 1.0 | 1.25A | 22 | 5A |  | USB-C |
| LP-IMU | 5V bus → IMU | 0.1 | 0.11 | 0.14A | 24 | 3.5A |  | WAGO/UART |
| ENC-01-04 | 5V bus → Encoders x4 | 0.01 ea | 0.011 | 0.014A | 26 | 2.5A |  | XT30/XT60 |
| SEC-01 | Sec Batt → BD-01/02 | 3 | 3.3 | 4.1A | 18 | 10A | 10A blade | XT30/ XT60 |
| BD01-OUT | Buck 12V → Jetson + LiDAR | 1.3 | 1.43 | 1.8A | 20 | 7.5A |  | Barrel 5.5/2.1 |
| BD02-OUT | Buck 5V → Router | 3 | 3.3 | 4.1A | 20 | 7.5A |  | USB-C |
| SIG-xx | Jetson → Controllers | <0.50 |  |  | 24 | 3.5A |  | Dupont.JST |

# Power budget

The rover uses a dual-source power architecture to separate the drivetrain and excavation subsystems from the computational subsystem. To prevent massive current spikes and thermal damage, drivetrain and excavation subsystem enforces a mutually exclusive operational rule that the excavation arm must never run while the drivetrain is active.

## Drivetrain and excavation subsystem:

The absolute maximum load is dictated by which part of the subsystem is be used:

### Scenario A: Driving & Navigating

In this scenario 4 drivetrain motors (GR-WM4-V3) would be utilised with each of them having a rated current of 3.8A, having a theoretical nominal current draw of 15.2A. The max current draw that the motor can reach is 22.6A according to the datasheet however at 13A the internal gearbox will break and therefore the Sabertooth 2x32 current limiting is configured in software to 10A per channel to enforce this gearbox limit. This means that the max theoretical current draw that the drivetrain will produce is 40A.

### Scenario B: Excavation (Rover is stationary)

In this scenario, 1 excavation motor (57BLR50-24-01-HG100) and 4 actuators would be utilised. The excavation motor has a rated current of 5.0A, and the actuators have a combined nominal current draw of approximately 6.4A, resulting in a theoretical nominal current draw of 11.4A. To protect the hardware, the excavation BLDC controller is protected by a 10A fuse, and the actuator motor controllers (Cytrons) are protected by their own 15A fuses. This ensures that while the nominal draw is low, the maximum theoretical current draw the entire excavation subsystem can pull during a severe mechanical jam is strictly physically capped at 40A before the local fuses blow.

## Computational subsystem:

The computation domain is supplied by a secondary 14.8V LiPo battery. This power is stepped down to 12V for the Jetson and stepped down to 5V for the sensors, fully isolated from any 40A motor spikes.

| Component | Max Power draw (W) | Current draw (A) |
| --- | --- | --- |
| Jetson | 25 | 2.25 (Max 3) |
| Cameras | 12.5 x 2 | 1.13 (Max 2) x 2 |
| Router | 6.5 | 0.59 (Max 1) |
| LiDar | 6.0 | 0.54 (Max 2) |
| Total | 50 | ~6A (Max 10) |

## Battery C-Rating Numerical Justification

The primary battery selected for the actuation domain is the Turnigy 8000mAh 6S LiPo with a 15C discharge rating.

Battery Capacity (C): 8000mAh = 8Ah.

Rated Constant Discharge: 8Ah * 15C = 120A.

Absolute Peak System Demand: 40A (Drivetrain capped).

The Safety Factor (SF) for the battery's discharge capability is calculated as:

With Sabertooth software current limiting enforced (10A per channel), the motive battery comfortably supplies the capped peak demand with a 3× safety factor. The LiPo cells will experience virtually zero thermal stress or voltage sag.

## Secondary Compute Battery (14.8V) C-Rating Justification

The secondary battery powering the autonomy and sensor stack is an 14.8V 5000mAh 4S LiPo with a 40C discharge rating.

Battery Capacity (C): 5000mAh = 5Ah.

Rated Constant Discharge: 5Ah * 40C = 200A.

Absolute Peak System Demand: 12A (BD-01 rail ~6A + BD-02 rail ~6A)

This gives a SF of:

The secondary battery is vastly over-specked for current delivery, capable of outputting nearly 20 times the maximum required amperage of the compute domain. This massive headroom is intentionally designed to guarantee absolute voltage stability (zero voltage sag) for the sensitive NVIDIA Jetson and onboard sensors, completely isolating them from the extreme 40A transients occurring on the mechanical actuation rail.

## Battery Runtime Calculation

Runtime is calculated using the highest continuous load profile (driving) and a safe discharge limit of 80% to protect the lithium cells.

Primary Actuation Battery (Turnigy 8000mAh / 8Ah):

Usable Capacity (80%): 8Ah *0.8 = 6.4Ah

Average Continuous Load: 15.2A

Runtime Calculation: (6.4Ah/15.2A) *60 minutes = 25.3 minutes

Secondary Compute Battery (Turningy 5000mAh / 5Ah):

Usable Capacity (80%): 5Ah * 0.8 = 4Ah

Continuous Compute Load: 6A

Runtime Calculation: (4Ah / 6A) *60 minutes = 40 minutes

# Physical integration

## Overview

This report summarises the design, packaging, and thermal considerations for the rover’s electrical enclosure. The objective of the enclosure design is to ensure safe power distribution, adequate thermal management, and maintainability while integrating electrical components within the mechanical constraints of the robot chassis.

The electrical box houses the primary computing unit, motor drive controllers, batteries, power distribution boards (PDBs), and supporting electronics. The design has been validated through CAD integration with the mechanical team to ensure correct mounting, airflow management, and serviceability.

## Electrical Layout Architecture

The enclosure follows a single-direction airflow layout to optimise cooling and minimise thermal interaction between sensitive electronics and high-power components.

Component zoning:

Cool Zone (Air Intake Region)

- Jetson Orin Nano (primary compute)

- Low-voltage electronics

- Power distribution board

Neutral Zone (Central Region):

- 22.2 V battery

- 14.8 V battery

- Batteries mechanically secured on the polycarbonate base plate

Hot Zone (Air Exhaust Region)

- 5 Motor Drivers

  - 2 motor controllers for drive train

  - 1 excavation motor controller

  - 2 motor controllers for actuators

- High-power PDB distributing power to motor drivers

This arrangement ensures the hardware receives the coolest incoming air, while the highest heat-generating components are located near the exhaust vents.

## Mechanical Packaging

The enclosure baseplate is constructed from 5mm polycarbonate, which serves as both the structural mounting surface and a passive heat spreader for power electronics.

Mounting strategy:

| Component:; Mounting Method:. Jetson Orin Nano; 10–20 mm standoffs. Motor Drivers; 10–15 mm standoffs. Power Distribution Boards; 8–12 mm standoffs. Batteries; Mechanically strapped. |  |
| --- | --- |

Clearance beneath PCBs allows:

- airflow circulation

- solder joint protection

- cable routing

Component Weight Summary: Electrical box contents (batteries, compute, motor drivers, wiring) ≈ 3.2 kg. Total electrical system including all external components (LiDAR, router, cameras, actuators ×4, drivetrain motors ×4, excavation motor) ≈ 9.4 kg. Motor and actuator values are estimated and should be replaced with measured figures once hardware arrives.

## Thermal Management Strategy

The enclosure utilises active airflow cooling the system relies entirely on the standoffs and active airflow for cooling.

Cooling system features:

- Filtered intake vents to reduce dust ingress in the competition environment.

- Cooling fans to maintain positive airflow.

- Exhaust vents positioned near motor drivers.

- Polycarbonate baseplate for strength.

Airflow path:

Filtered Intake → Jetson & Low Power Electronics → Battery Region → Motor Drivers → Exhaust

This layout prevents heat generated by motor drivers from recirculating toward sensitive computer hardware.

## Electrical Safety Considerations

The electrical system incorporates several safety measures:

- Separate batteries for compute and drive systems

- Dedicated power distribution boards

- Battery mounting with insulation and mechanical retention

- Short high-current cable routing between batteries and PDBs

This configuration improves reliability and simplifies fault isolation during operation.

## CAD Integration & System Validation

The electrical box layout has been integrated with the mechanical team’s CAD assembly to confirm:

- All components are mounted correctly

- Cooling airflow paths are unobstructed

- No packaging conflicts exist

- Serviceability and maintenance access are maintained

## Evidence

- Electrical box layout developed and validated in CAD

- Cooling strategy reviewed with mechanical team

- Final packaging design confirmed for component clearance and airflow

Figure 5 CAD Files for Electrical Box

Figure 6 Exhaust and Cooling Diagram

# E-STOP & safety chain

## Back EMF

As relating to safety, it can be considered that back EMF can result in potential hazards, as some components are not designed to handle them. To analyse and eliminate threats, the diagram must be understood. There are 2 voltage sources one on the far left and one on the far right. The far left (main battery) provides 22.2V handled by a 8,000 mAh and the far right (secondary battery) provides 14.8V.

The heavy lifting is carried out by the main battery, as it provides a greater amperage required for high torque applications with the motors and servos, covering majority of the mechanical motion. The secondary battery handles all input and processing power and is smoothened by the power distribution board. All power links go into the Jetson which then powers the other sensors. At no point does the powering of the main battery and the secondary battery intersect, and only signal wires are present to provide data via PWM pins.

The PDB listed between the secondary battery and the Jetson Orion nano, does not handle back EMF, but is not required to do so, as the Jetson has no physical component to allows back flow. The other sensors connected cannot cause back flow either.

The servos and motors have potential cause for backflow, provided a slippage occurs when the rover is stationary, and the motors could provide a generator effect. In order to handle these motors, the drivers selected have their own unique features. 2 of the 3 have regenerative braking, designed to handle backflow and use braking to regenerate power. The cheaper option does not have regenerative braking but has basic protection enough to handle back EMF in this situation. Additionally, diodes can be introduced with resistors in place to handle excessive current spikes.

In conclusion, there is potential for back flow to occur, but is handled and additional support can be provided if testing does not go according as expected.

## Bench Test

As we do not have the physical components with us right now, a bench test can be confirmed by utilizing sources to showcase something of similar insight has been before and justified about how it a predictable outcome will arise.

Hardware Component Overview — FIRST Robotics Competition documentation

The link above showcases the basics circuits used in FRC (FIRST robotics competition). The battery is connected to a 120A main breaker, ideally this would be a relay switch, where a smaller current is handled by a switch which in turn cuts off a high current line. The flow diagram showcased has its battery connected in parallel, and there is no voltage leaks between either battery listed. A single E-stop will be placed exclusively on the 22.2V power domain. When triggered, it instantly severs all power to the actuators and drivetrain, while leaving the independent 14.8V compute battery untouched. This ensures the Jetson and router safely stay powered on, so telemetry and video feeds are not lost during a safety stop.

## E-stop requirements

The E-stop selected for INNEX-1 is a Heavy-duty DC emergency stop switch — 250 A @ 80 V DC, push-break/pull-make, M8 copper posts, IP50, 10,000 cycle life. Meets competition requirements for a latching push-to-stop switch.

The rule book says the button must be an unmodified commercial off the shelf red button and must have minimum diameter of 40mm.

It must disable power with a single push motion and latch in a stop position until it is intentionally undone via a twist or pull action.

While a mechanical relay (contactor) circuit is permitted, utilizing a direct high-current disconnect switch like the ADK 125A reduces wiring complexity, eliminates the risk of relay-coil failure, and provides a more robust physical break in the 22.2V circuit.

To comply with competition safety standards, the E-stop will be wired as the first component in-line immediately following the battery's main positive terminal, completely isolating the power distribution board when pressed.

So, the following buttons are viable that follow these rules:

Main Choice: Heavy-duty DC emergency stop — 250 A @ 80 V DC (installed) – quote requested

Alternative Choice: ED125-4 | Albright 125A at 48V Battery Isolator Stop Switch – quote requested

## E-Stop Schematic

Figure 7 E- Stop and Wiring Schematic

# Procurement locked

## Main parts BOM

| No# | Component name | QTY. | Price | Notes | Sub-system | Companies/Suppliers | Shipping Fees | Delivery |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Cameras | 2 | £301 |  | Sensors | Luxonis | DHL £15 | 2-5 days DHL |
| 2 | Main battery | 1 | £68.3 |  | Power Systems | Turnigy | £11.37 | 3-7 Days, Free Delivery if we get both batteries |
| 3 | Secondary battery | 1 | £34.28 |  | Power Systems | Turnigy | £6.72 | 3-7 Days, Free Delivery if we get both batteries |
| 4 | Drivetrain motor (GR-WM4-V3) | 4 | £75.5 |  | Power Systems | GimsonRobotics | None | Secured via direct supplier partnership (Awaiting dispatch) |
| 5 | Excavation motor | 1 | £46.79 |  | Power Systems | StepperOnline | £18.14 | £26.74; 4-10 working days. |
| 6 | Voltage regulator 1 | 1 | £3.22 |  | Power Systems | Switch Electronics | None | £3; 2-3 working days |
| 7 | Voltage regulator 2 | 1 | £7.77 |  | Power Systems | Amazon | None | 2-3 days |
| 8 | Actuators | 4 | £67.75 |  | Power system | TRU COMPONENTS | £45.17 | 50mm TRU ×2 (22V, Cytron #1) + 250mm DCHOUSE ×2 (12V, Cytron #2). No fees, 4 days |
| 9 | Power Distribution Board | 1 | £31.28 |  | Power Systems | Amazon | None | 1 week |
| 10 | Microcontroller | 1 | £280.6 | NVIDIA Jetson Orin Nano Super Developer Kit | Control Systems | RS | None | Free, 2-3 working days |
| 11 | Sabertooth Motor controllers | 2 | £114.1 | 2 for drivetrain | Control Systems | Robotshop | None | £25.59 (Free if we get Cytrons); 2-7 working days |
| 12 | Cytron Motor Controllers | 2 | £20.03 | 2 for actuators | Control Systems | Robotshop | None | £25.59 (Free if we get Sabertooths); 2-7 working days |
| 13 | BLD-510B BLDC Motor Controller | 1 | £12.54 | 1 for excevation | Control Systems | Amazon | None | £0.79; 1-2 weeks |
| 14 | Dual-band router | 1 | £55.24 |  | Communication System | Amazon | None | £5; 1 week |

## Cost

Total: £2,172.28

Electrical total: £1,552.29

Sponsorship (Luxonis): £376

Electrical total (after discount): £1,176.29

Shipping Fees Total: £96.4

Shipping Fees Electrical: £33.09

Delivery Fees Total: £35.53

Delivery Fees Electrical: £9

Total (including shipping and delivery): £1927.16

Total Electrical (including shipping and delivery): £1218.38 (without LiDAR: £1029.58)

## Alternative List

| Sub-system | Primary Component | Alternative Component | Alternative Cost | Trade-off / Justification |
| --- | --- | --- | --- | --- |
| Control Systems | NVIDIA Jetson Orin Nano | Raspberry Pi 5 (8GB) | £80.00 | If the Jetson fails or faces supply chain issues, the RPi 5 is a viable backup. Loss of onboard CUDA cores. However, because we utilize Luxonis OAK-D cameras, the RPi 5 has enough CPU overhead to successfully run Nav2 and our ROS 2 partial autonomy. |
| Control Systems | Sabertooth Dual Motor Controller | RoboClaw 2x60A | £125.00 | Direct competitor to Sabertooth. Slightly higher cost but offers dual-channel DC motor control with built-in USB/Serial ROS 2 compatibility and thermal protection for the 22.2V power domain. |
| Control Systems | Cytron Motor Controllers | BTS7960 43A Motor Driver | £10.00 | Backup for the linear actuators. Very cheap driver but lacks the logic-level protection of the Cytron. Requires slightly more complex logic from the Jetson to prevent short circuits. |
| Power Systems | 250A DC E-Stop Switch (push-break, 80V, M8 posts) | Albright ED125-4 | £45.00 | The Albright is the gold standard for heavy industrial battery isolation. Higher cost and longer lead time than the ADK but guarantees the same 125A breaking capacity to safely break a 60A DC rail without problem. |
| Communication | Slate Plus Dual-Band Router | GL.iNet Beryl AX (GL-MT3000) | £75.00 | Available on Amazon. Higher cost but provides Wi-Fi 6 bandwidth to easily keep our compressed telemetry data under the rulebook's 4,000 Kbps limit. |
| Sensors | Ouster OS1 LiDAR | Livox Mid-360 | £550.00 | The industry standard for 3D SLAM on robots. Provides dense 360-degree point clouds fully capable of mapping negative obstacles (craters) in the lunar simulant. Drastically higher cost. The Ouster OS1 was uniquely selected for its sub-£200 price point. Implementing the Livox backup would instantly break the budget cap, requiring a budget extension or an external LiDAR sponsorship. |